STILMAT  MAGNAT – Rubber/Carpet - open

Text for invitation to tender

| Description: | Flexible and roll-up, aluminum entrance mat for laying on the floor. Custom produced mats to fit width and walking depth specifications perfectly. Suitable for wheeled traffic. Any geometric shape is available on demand. |
| --- | --- |
| Model | MAGNAT (Rubber & Carpet) |
| Tread surface (inlay): | Profiled rubber inserts and high quality dust control carpet, durable and luxurious character. Made from 100% Polyamide |
| Ideal for | Coarse dirt, fine dirt and moisture |
| Colors: | Black, Grey, Brown… |
| Version | Open system |
| Code | CORC |
| Approx. height (mm) | 20 mm |
| Entrance position | Covered outdoor areas, gateways, passages, indoors |
| Zones: I II III | II |
| Load | Normal |
| Footfall | Daily footfall up to 1000 |
| Roll-over and drive-over capability: | Wheelchairs, Prams, Hand and Light shopping trolleys |
| Support chassis | Made from rigid aluminum with rubber sound insulation underlay |
| Aluminum material thickness (mm) | 1.0 |
| Connection | Strong and flexible PVC connection strip, the entire width |
| Standard profile clearance approx. (mm) | 7.5 mm |
| Slip resistance | R 11 slip resistance as per DIN 51130 |
| Logo options | Logo engraving |
| Contact | STILMAT |